# Week 2 – Network & Communication Security (Short Notes)

## TCP/IP Layers (Simple)

- Physical – cables, Wi-Fi signals (just bits on a wire).

- Data Link – MAC addresses, local network delivery.

- Network (IP) – routing across networks.

- Transport –

  - TCP: reliable, ordered delivery (3-way handshake).

  - UDP: fast, no checks (streaming/gaming).

- Application – protocols like HTTP, DNS, SMTP, FTP.

## Key Networking Protocols

### DHCP

Automatically gives devices:

- IP address

- Subnet mask

- Gateway

- DNS

### DNS

Translates domain names → IP addresses.

### ARP

Finds the MAC address that matches an IP (local network only).

### TCP

Reliable connection:

SYN → SYN-ACK → ACK

### UDP

Fast, “fire-and-forget,” no reliability.

## Cryptography Basics

### Symmetric Encryption

- Same key → encrypt + decrypt.

- Fast.

- Example: AES.

### Asymmetric Encryption

- Public key = encrypt

- Private key = decrypt

- Used for key exchange + digital signatures.

- Example: RSA.

### Hashing

- One-way, irreversible.

- Ensures integrity.

- Example: SHA-256.

## TLS / HTTPS

TLS encrypts web traffic.

Basic handshake:

- ClientHello

- Server sends certificate

- Client verifies

- Secure key shared

- Encrypted session begins

## PGP (Email Security)

Hybrid method:

- Encrypt message with symmetric key (fast).

- Encrypt the key with receiver’s public key (secure).

- Sign with sender’s private key (authenticity).

## IPSec (VPN Security)

### Two components:

- AH = integrity + authentication

- ESP = encryption + integrity

### Modes:

- Transport mode – host ↔ host

- Tunnel mode – gateway ↔ gateway (VPNs)

## Key Takeaways

- DHCP = gives IP

- DNS = name → IP

- ARP = IP → MAC

- TCP = reliable

- UDP = fast

- TLS = secure web

- IPSec = secure networks

- PGP = secure email

- Hashing = integrity

- Encryption = confidentiality

Extended Reading:

PGP Intro (Pretty Good Privacy) - Adrian Perrig Page

What PGP is and why it was developed are explained in this reading. A secure email and file protection program is called PGP (Pretty Good Privacy). It combines symmetric and asymmetric cryptography: the communication is encrypted using a quick symmetric key, and only the recipient's private key may decrypt the symmetric key. This provides you with integrity (you can detect modifications), secrecy (only the intended person can view it), and digital signatures for verification and non-repudiation. Instead of relying on a single central certificate authority like SSL/TLS, PGP uses a "web of trust," in which users sign each other's keys and trust is developed socially rather than by a single large corporation or root CA.

AWS Page

This reading explains what PGP is and why it was created. PGP (Pretty Good Privacy) is a secure email and file protection application. It mixes symmetric and asymmetric cryptography: a rapid symmetric key is used to encrypt the communication, and the symmetric key can only be decrypted with the recipient's private key. This gives you digital signatures for verification and non-repudiation, integrity (you can detect modifications), and secrecy (only the intended person may read it). PGP uses a "web of trust," where users sign each other's keys and trust is built socially rather than by a single big company or root CA, as opposed to depending on a single central certificate authority like SSL/TLS. This reading explains what PGP is and why it was created. PGP (Pretty Good Privacy) is a secure email and file protection application. It mixes symmetric and asymmetric cryptography: a rapid symmetric key is used to encrypt the communication, and the symmetric key can only be decrypted with the recipient's private key. This gives you digital signatures for verification and non-repudiation, integrity (you can detect modifications), and secrecy (only the intended person may read it). PGP uses a "web of trust," where users sign each other's keys and trust is built socially rather than by a single big company or root CA, as opposed to depending on a single central certificate authority like SSL/TLS.